My response is italicized.

Original Text (Chapter 2, Section 2.3.1)

“In this analysis, the hourly spatial medians of the 9 measured variables are log-transformed rather than log-difference-transformed as was done previously in Melnikov et al.28 PCA is sensitive to outliers and that some transformation is necessary; however, conventional environmental practice dictates log transformation rather than log differencing.16 EVs returned by log-transformed observations are also consistently higher than the log-differenced-transformed observations, as shown in Figure 2.”

Discussed during the defense, but the median is more resistant to outliers. In this case, I argue it’s appropriate over other choices as the objective of the analysis is to evaluate region-wide trends in air quality. The median would minimize the impact that any localized influence could have on any one monitor. However, in other analyses, other statistics would be more appropriate. The text has been revised to the following.

Revised Text:

“We follow Melnikov et al. and take the spatial median instead of some other statistic, such as the mean. Medians are more outlier resistant compared to the mean. Since our objective is to understand area-wide trends in air quality, using the median minimizes the impacts of local air pollution events at any given monitor compared to a statistic such as the mean. However, in different contexts, such as health impact studies or analyses of extreme events, use of other statistics such as the mean may be more appropriate.

In this analysis, the hourly spatial medians of the 9 measured variables are log-transformed rather than log-difference-transformed as was done previously in Melnikov et al.28 PCA is sensitive to outliers and some transformation is necessary; however, conventional environmental practice dictates log transformation rather than log differencing.16 EVs returned by log-transformed observations for the first component are also consistently higher than the log-differenced-transformed observations, as shown in Figure 2. This suggests that a more parsimonious representation is captured in log transformation rather than in log-differenced transformation. “

Original Text (Chapter 3, Section 3.3.1):

“Another potential bias less discussed in the mobile monitoring literature is the intra-census tract temporal bias. This is the bias that occurs from sampling within a census tract. Intra-census tract bias could occur when one samples a highly temporally variable pollutant, such as O3, within a census tract starting at a fixed given location. Since sampling census tracts is a process that takes place on the order of hours and that some photochemical pollutants such as O3 can experience significant concentration changes on smaller time scales, it follows that spatial differences in concentration within the census tract can present themselves as a function of time rather than space alone.

While correction factors that incorporate stationary monitor measurements could ostensibly address intra-census tract bias,53 such an approach suffers from questions regarding the applicability of stationary measurements to measurements far removed from the monitor. In this work, a time-series based method to estimate the intra-census tract temporal bias for all census tracts analyzed was employed.  The method involved calculating the median change in rolling 5th percentile across a 10-minute time window of mobile pollutant concentrations across all census tract drive periods for a given census tract. Estimates of this census tract intra-temporal bias are given in Table 4 when applicable. While calculated biases for NO2 and UFP were small, these values could become more significant for more temporally variable pollutants such as O3 and present challenges in their analysis in mobile monitoring contexts.”

This comment made me realize that temporal sensitivity was an inappropriate term for what I wanted to describe. That description has been updated below.

Revised Text:

“Another potential bias less discussed in the mobile monitoring literature is the intra-census tract temporal bias. This is the bias that occurs from sampling within a census tract. Intra-census tract bias could occur when one samples a pollutant, such as O3, within a census tract starting at a fixed given location. O3 is an example of a pollutant which can exhibit significant, prolonged concentration changes over large temporal and spatial scales.53 Since sampling census tracts is a process that can take place on the order of hours, gradients of a pollutant such as ozone can present themselves as a function of both time and space rather than space alone.

While correction factors that incorporate stationary monitor measurements could ostensibly address intra-census tract bias,54 such an approach suffers from questions regarding the applicability of stationary measurements to measurements far removed from the monitor. In this work, a time-series based method to estimate the intra-census tract temporal bias for all census tracts analyzed was employed.  The method involved calculating the median change in rolling 5th percentile across a 10-minute time window of mobile pollutant concentrations across all census tract drive periods for a given census tract. Estimates of this census tract intra-temporal bias are given in Table 4 when applicable. While calculated biases for NO2 and UFP were small, these values could become more significant for other pollutants such as O3 and present challenges in the analysis.”

Original Text (Chapter 4, Introduction):

“The complexities in understanding and defining background have led to a multitude of published techniques which aim to identify and quantify this signal in mobile monitoring contexts. The plethora of these techniques poses a reproducibility problem, first noted by Brantley and followed-up six years later.50,67 This chapter proposes another background identification and quantification technique which attempts to address the background problem in mobile monitoring time series and aspires to be more robust than previously published approaches. The approach utilizes a time series regime change modeling technique, Hidden Markov Models (HMMs), and utilizes decoded states to inform a spline fit that becomes the modeled background curve for a day’s worth of pollution measurements. In this chapter, the methodology, its validation, and results of its application on the Houston mobile monitoring dataset are discussed. This methodology recasts the background detection problem as the establishment of a statistical baseline and deviations from that baseline and gets away from the concept of background as a signal derived from arbitrary time windows based on assumptions of air pollution spatial distributions that become more flawed in more complex mobile monitoring time series. “

Attempted to make a physical, more intuitive explanation with the updated text below:

Revised Text:

“The complexities in understanding and defining background have led to a multitude of published techniques which aim to identify and quantify this signal in mobile monitoring contexts. The plethora of these techniques poses a reproducibility problem, first noted by Brantley and followed-up six years later.50,68 This chapter proposes another background identification and quantification technique which attempts to address the background problem in mobile monitoring time series and aspires to be more robust than previously published approaches. The approach utilizes a time series regime change modeling technique, Hidden Markov Models (HMMs), and utilizes decoded states to inform a spline fit that becomes the modeled background curve for a day’s worth of pollution measurements. HMMs systematically categorize time series realizations as belonging to one state or another based on whether those realizations exhibit similar statistical behavior to one another. In this chapter, the methodology, its validation, and results of its application on the Houston mobile monitoring dataset are discussed. This methodology recasts the background detection as envisioning the mobile platform traversing through a series of clean regimes and dirty regimes, with no fixed time constraints governing how long the mobile platform is in either regime, and fits a signal to measurements made in the clean regime. The methodology rejects conceptualizing background as a signal derived from static time windows based on assumptions of source spatial distribution and duration that become more flawed in more complex mobile monitoring time series.”

Original Text (Chapter 4, Section 4.2.1):

“HMMs describe a class of time series regime change models that have been employed in finance, signals processing, and social sciences applications.69,70 The basic premise of an HMM is that a time series can be broken up into a series of observations whose probability distributions are governed by hidden states. These hidden state sequences, in conjunction with the parameters of the probability distributions they govern, are estimated via maximum likelihood estimation. HMM states are powerful tools that can be decoded to provide crucial information about a time series. For example, Langrock et al. illustrate how white tip shark accelerometer data could be described by three different categories of shark behavior.69”

I’ve written in an example of a coin-flip trial below to help build intuition:

Revised Text:

“HMMs describe a class of time series regime change models that have been employed in finance, signals processing, and social sciences applications.70,71 The basic premise of an HMM is that a time series can be conceptualized as a series of observations whose probability distributions are governed by hidden states. Consider the hypothetical example of an individual flipping two coins, one that is unbiased, and the other with “heads” on both sides. An observer witnesses this individual flipping either of these two coins five times in succession. The observer does not know which coin has been flipped; all that is known are the outcomes of each flip (e.g. H,T,H,H,H). In the HMM framework, each flip is governed by a hidden state sequence consisting of the two coins: the unbiased coin, or the biased coin. These hidden state sequences, in conjunction with the parameters of the probability distributions they govern, are estimated via maximum likelihood estimation. HMM states are powerful tools that can be decoded to provide crucial information about regimes in a time series. As an example, Langrock et al. illustrate how white tip shark accelerometer data could be described by three different categories of shark behavior.70”

Original Text (Conclusion):

## Implications

I’ve rewritten the chapter to include more lengthy sections detailing future steps I see for each section.

Revised Text:

## “Implications and Future Directions

This thesis develops novel statistical methods to provide atmospheric scientists with more insight into the measurements they make. Chapter 2 discusses DPCA and how applying PCA on a rolling time window addresses shortcomings in assumptions made about atmospheric data. Chapter 3 gives an overview of the Houston mobile monitoring campaign and discusses some preliminary work to make measurements more representative of characteristic concentrations over the 9-month campaign interval. Chapter 4 discusses the need for replicability in background detection and removal and describes the development of a novel algorithm, SIBaR, which incorporates HMMs to model regime changes in mobile monitoring time series. Chapter 5 discusses plume detection in the context of anomaly detection and describes a novel algorithm incorporating DBSCAN to detect plumes within mobile monitoring time series, highlighting stark disparities in air pollution exposure that should be addressed in future research.

Chapter 2 discusses a solution to potential pitfalls users of dimension reduction techniques such as PCA and PMF encounter when using these techniques on atmospheric data. Most atmospheric data are inherently non-stationary and exhibit non-linear behavior, making linear models unsuitable for air quality data taken on more extensive time scales. A solution aimed at PCA, first formulated by Melnikov et al.,28 applies PCA on a rolling time window where the time window can be set to increments where atmospheric data can be expected to display linear behavior. More importantly, DPCA captures time dependency, revealing diurnal and seasonal patterns that would not be apparent in a conventional PCA application. Future efforts would explore applications of these components as inputs into other statistical models, such as artificial networks, as well as integration of these components with other atmospheric modeling efforts to interpret these components. As an example, derived dynamic EVs could be linked with potential source contribution functions93 and other wind trajectory analyses to infer meaning behind derived components.

Chapter 3 defines mobile monitoring and gives an overview of the Houston mobile monitoring campaign. It discusses work done to process large amounts of air pollution data for 7 different air pollutants collected in a 9-month time span in different Houston census tracts. The chapter then discusses philosophical issues with mobile measurements, and describes work done to address intra-census tract biases, implementing extra aggregation steps to avoid oversampling on mobile visits, and a correction to a data-based subsampling routine used to determine representativeness of mobile measurements. It concludes with an illustration of the power of these mobile measurements, highlighting published work illustrating air pollution hotspot identification around Houston. Future work should incorporate mobile measurements to understand air pollution on finer spatial scales than what is possible with stationary monitors.

Chapter 4 talks about replicability issues associated with estimating the background in mobile monitoring time series and offers a new technique that could serve as the framework for constructing a more consistently used estimate for background. The technique, coined SIBaR, incorporates HMMs into partitioning daily mobile monitoring time series into background points and source influenced points, then fits a spline to background identified points. The technique produced similar results to another published technique, but differed in several key aspects that, along with addressing its shortcomings in computational time and sensitivity to noise, would merit its continued use. Future work for SIBaR should address its shortcomings, namely its sensitivity to instrument noise and error (such as instrument drift) and the infeasibility of linearity assumptions currently embedded in the model. Steps within the algorithm can be developed to flag periods of instrument drift, as an example, and the user could be given a suite of options to determine how to address it. Formulating response distributions as a linear combination of spline basis functions80 is a promising next step in addressing shortcomings with linearity assumptions embedded in the model and will expand use cases for SIBaR.

Chapter 5 recasts the plume detection problem in mobile monitoring time series as an anomaly detection problem and argues for its use in determining source influences on mobile monitoring time series. It incorporates DBSCAN, a density-based clustering technique, into an algorithm that systematically labels plumes within mobile monitoring time series as anomalies. Performance of this algorithm was compared with several other time-series based plume detection algorithms and found to exhibit superior performance in not labeling normal points as anomalous, while it predicted fewer anomalies successfully compared to another base-case algorithm. The anomalies were then taken, clustered, and probed for significance through evaluating relationships to traffic related variables which were then matched with previously published studies. The chapter concludes by showcasing the spatial disparities in these different anomaly types across the city of Houston, calling for further research into trucking-related emissions in heavily-industrialized neighborhoods. Future work focused on the method itself should expand to using it with other pollutants, such as carbon monoxide to provide another marker for light duty vehicle activity. It could also incorporate sulfur dioxide to distinguish plumes from industrial activity. Future work could explore further validation of clustered anomaly patterns by including an observer to witness air pollution events and confirm that the interpretation of these clusters coincides with observations. Our work largely focused on mobile sources, such as light-duty and heavy-duty vehicles, but other sources, such as trains, could also play a role. Other work could focus on improving performance of the algorithm itself, either by exploring other statistical measures to identify anomalies in data, such as local outlier factors, or by incorporating DBSCAN with other anomaly detection algorithms in an ensemble approach.

While the statistical techniques in this thesis largely focused on mobile measurements, some of these techniques can be applied to stationary monitor air pollution measurements. HMMs, featured in the SIBaR partitioning step, already have several published use cases in stationary monitor networks.94 The plume detection algorithm could facilitate rapid identification of plumes in stationary monitor data. One could explore developing an automated emergency monitoring response system, in which the plume detection algorithm could identify plumes in stationary monitor data and deploy mobile monitors to the area around the stationary monitor which triggered the warning for more fine scale monitoring.

It is an exciting time to be an atmospheric scientist. The field is rapidly becoming data dependent, with refinement in stationary monitoring networks and development of new ones, the proliferation and development of low cost air pollution sensors,95 and the development of mobile platforms to take measurements at high spatial resolution. With the development of all these new measurement platforms comes the requirement to develop new statistical techniques to analyze the measurements they make. This thesis serves as a step in that direction.”